# Low Sulfur Fuel Oil (Futures) Inspection Rules (Trial)

## Chapter 1	General Provisions

Article 1	These Rules are jointly formulated by the low sulfur fuel oil futures inspection agencies designated by the Shanghai International Energy Exchange (“INE” or the “Exchange”).

Article 2	These Rules are formulated in accordance with national provisions on commodity inspection and applicable rules of the Exchange to ensure the smooth inspection for low sulfur fuel oil futures delivery and to standardize the inspection prior to physical delivery.

Article 3	These Rules are applicable to the inspection for low sulfur fuel oil futures delivery. The Exchange, Designated Inspection Agencies, Designated Delivery Storage Facilities, consigners of inspection (“Consigners”), and other relevant organizations shall abide by these Rules.

## Chapter 2	Inspection Procedures

### Section 1	Inspection Standards and Methods

Article 4	Inspections under these Rules shall be governed by the standards and methods set forth in the following documents. For undated references, the latest edition of the referenced document shall apply:

- GB/T 1885 – Petroleum Measurement Tables

- GB/T 4756 – Method for Manual Sampling of Petroleum Liquids

- GB/T 6041 – General Rules for Mass Spectrometric Analysis (for measuring the amount of styrene and phenol)

- GB/T 8927 – Petroleum and Liquid Petroleum Products - Temperature Measurement - Manual Methods

- GB/T 13236 – Petroleum and Liquid Petroleum Products - Equipment for Measurement of Liquid Level in Storage Tanks - Manual Methods

- GB/T 13894 – Petroleum and Liquid Petroleum Products - Measurement of Liquid Level in Tanks - Manual Methods

- GB/T 19779 – Petroleum and Liquid Petroleum Products – Calculation of Oil Quantities - Static Measurement

- JJG 168 – Verification Regulation of Vertical Metal Tank Capacity

- API MPMS 17.6 – Guidelines for Determining the Fullness of Pipelines between Vessel and Shore Tanks

- ASTM D93 – Standard Test Methods for Flash-Point by Pensky-Martens Closed Cup Tester

- ASTM D95 – Standard Test Method for Water in Petroleum Products and Bituminous Materials by Distillation

- ASTM D97 – Standard Test Method for Pour Point of Petroleum Products

- ASTM D240 – Standard Test Method for Heat of Combustion of Liquid Hydrocarbon Fuels by Bomb Calorimeter

- ASTM D445 – Standard Test Method for Kinematic Viscosity of Transparent and Opaque Liquids (and Calculation of Dynamic Viscosity)

- ASTM D482 – Standard Test Method for Ash from Petroleum Products

- ASTM D664 – Standard Test Method for Acid Number of Petroleum Products by Potentiometric Titration

- ASTM D1298 – Standard Test Method for Density, Relative Density, or API Gravity of Crude Petroleum and Liquid Petroleum Products by Hydrometer Method

- ASTM D4294 – Standard Test Method for Sulfur in Petroleum and Petroleum Products by Energy Dispersive X-ray Fluorescence Spectrometry

- ASTM D4530 – Standard Test Method for Determination of Carbon Residue (Micro Method)

- ASTM D4740 – Standard Test Method for Cleanliness and Compatibility of Residual Fuels by Spot Test

- ASTM D4870 – Standard Test Method for Determination of Total Sediment in Residual Fuels

- IP 501 Determination of Aluminium, Silicon, Vanadium, Nickel, Iron, Sodium, Calcium, Zinc and Phosphorous in Residual Fuel Oil by Ashing, Fusion and Inductively Coupled Plasma Emission Spectrometry

- IP570 – Determination of Hydrogen Sulfide in Fuel Oil - Rapid Liquid Phase Extraction Method

### Section 2	Load-In Inspection

Article 5	Acceptance of Inspection Assignment by Designated Inspection Agencies

Consigner shall entrust, in writing and 24 hours before load-in, a Designated Inspection Agency to perform the load-in inspection (comprising quality pre-check before load-in as well as weight and quality inspections for the shore tanks for load-in), and file with it the load-in plan mutually agreed upon by the Consigner and the Designated Delivery Storage Facility. If the residual oil within the vessel-to-shore tank pipeline needs to be displaced, the Consigner shall submit the displacement plan at the same time and specify the pipeline displacement volume. The Designated Delivery Storage Facility shall displace the residual oil if it is of a lower quality than that of the low sulfur fuel oil to be loaded in for futures delivery.

The Consigner shall provide the following documents  for the purpose of entrustment:

(1) For inbound bonded low sulfur fuel oil to be delivered directly from the ships to the Designated Delivery Storage Facility after berthing and unloading: Low Sulfur Fuel Oil Inspection Certificate issued by the loading port, bill of lading, customs load-in approval document, low sulfur fuel oil load-in application / approval notice from the Exchange, and other documents as required;

(2) For inbound bonded low sulfur fuel oil to be delivered to the Designated Delivery Storage Facility through ship-to-ship transfer in a domestic anchorage: low sulfur fuel oil inspection certificate issued by the loading port of the mother ship, bill of lading, customs load-in approval document, fuel oil load-in application / approval notice from the Exchange, and other relevant documents. The Designated Inspection Agency shall supervise the unloading and loading of fuel oil during the ship-to-ship transfer, securely seal the tanks on the receiving vessel for pre-unloading verification at the Designated Delivery Storage Facility, and provide relevant supporting documents as required;

(3) For bonded low sulfur fuel oil under the processing trade route or the general trade route  to be delivered to the Designated Delivery Storage Facility: export customs declaration form, export verification checklist under the processing trade manual, export statutory inspection certificate, commodity inspection certificate of low sulfur fuel oil issued by the loading port, producer’s product quality inspection report, certificate of origin, low sulfur fuel oil load-in application / approval notice from the Exchange, and other materials as required.

(4) For bonded low sulfur fuel oil delivered to the Designated Delivery Storage Facility from other bonded storage facilities through customs transit or blended in the Designated Delivery Storage Facility: the commodity inspection certificate for the bonded blended low sulfur fuel oil, certificate of origin, low sulfur fuel oil load-in application / approval notice from the Exchange, and other materials as required.

The Designated Inspection Agency shall maintain close contact with the Consigner and the Designated Delivery Storage Facility, stay current with the load-in status, and timely arrange for inspections.

Article 6	On-Site Load-In Inspection by Designated Inspection Agencies

(1) Quality pre-check before load-in

(A) The Consigner shall, before the unloading and load-in of low sulfur fuel oil for futures delivery, determine with the Designated Inspection Agency and Designated Delivery Storage Facility the time, location, and method of sampling for the quality pre-check before load-in. Where the Consigner chooses to conduct the pre-check at a domestic loading port, the Designated Inspection Agency shall take samples from the ship tanks immediately after loading; then seal all compartment hatch covers and valves, small hatch covers, choke outlet valves, seawater valves, as well as outboard discharge valves; and record the seal numbers. Upon arrival of the vessel at the unloading port, the Designated Inspection Agency shall verify the integrity of the seals and the seal numbers. Unloading shall only be carried out if the shipment passes the quality pre-check. Where the Consigner chooses to conduct the pre-check and sampling at the unloading port, unloading shall only be carried out if the samples from the ship tanks pass the pre-check.

(B) Sampling standard: GB/T 4756.

(C) The Designated Inspection Agency shall carry out laboratory testing according to the scope of quality pre-check specified by the Exchange, which covers density, kinematic viscosity, sulfur content, moisture, flash point, and compatibility (where mixed tank storage is needed) using the standards and test methods set out in Appendix I.

(D) Upon completion of the quality pre-check, the Designated Inspection Agency shall issue a pre-check report and timely submit it to the Consigner, the Designated Delivery Storage Facility, and the Exchange. Unloading and load-in shall not begin until the positive pre-check report is received.

(2) Sealing and unsealing

(A) The Designated Inspection Agency shall ensure all valves to any shore tank not designated for futures delivery are tightly closed and properly sealed.

(B) The Designated Inspection Agency shall, together with the Designated Delivery Storage Facility, seal the outlet valves, water discharge valves, and drain valves of the shore tanks designated for futures delivery (“designated futures shore tanks”) as well as all bypass valves connected to the unloading pipelines, and record and submit the seal numbers to the Designated Delivery Storage Facility for signature confirmation, so as to ensure the designated shipment is fully unloaded into designated shore tanks.

(C) Upon completion of load-in, the Designated Inspection Agency shall, together with the Designated Delivery Storage Facility, verify the integrity of seals and seal numbers before unloading, and shall ensure the inlet valves to the designated futures shore tanks are closed and sealed; record and submit the seal numbers to the Designated Delivery Storage Facility for signature confirmation; and remove the seals on the bypass valves connected to the unloading pipelines.

(3) Pre-load-in shore tank inspection

(A) Measurement

(a) Ascertain the qualifications of the shore tanks designated for low sulfur fuel oil delivery set in JJG 168. Shore tanks shall be calibrated by a qualified national metrological administrative authority; the tank capacity tables shall be within the calibration validity period.

(b) Check the fullness of shore tank pipelines. The pipelines shall be fully filled, either by circulation for Designated Delivery Storage Facilities equipped with circulation facilities or by displacement of the residual oil in the vessel-to-shore tank pipelines, to ensure accuracy in the measurement of the quantity of low sulfur fuel oil transferred.

(c) Ascertain, in accordance with GB/T 13236, that all equipment used for shore tank measurement (including thermometers and gauging tapes, etc.) is calibrated by a qualified national metrological administrative authority, within the calibration validity period, and compliant with the explosion protection policies of the Designated Delivery Storage Facility.

(d) Measure the innage or ullage, free water, oil temperature, and tank ambient temperature together with the measuring personnel of the Designated Delivery Storage Facility and the Consigner in accordance with GB/T 13894 and GB/T 8927.

(e) If oil transfer is not started within eight (8) hours following the initial measurement, re-measurement according to the foregoing method shall be conducted and the results of which shall prevail.

(f) Measurements in shore tanks that have a low level of low sulfur fuel oil shall avoid the critical zone.

(B) Sampling

(a) Sample categories

(i) Sample A1: Sample from a single ship tank or single container (multiple samples)

(ii) Sample A2: Proportional mixture of A1 (two samples)

(iii) Sample B: Sample from shore tank before load-in (three samples)

(iv) Sample C: Sample from shore tank after load-in (three samples)

(b) Ship tank sampling (Sample A1)

(i) After boarding, request from vessel charterer the materials related to the loading of low sulfur fuel oil for futures delivery, including the certificate of weight, ullage report, bill of lading, certificate of quality, vessel documentation, piping layout, vessel experience factor, and stowage plan, among others.

(ii) Request from the charterer the accompanying low sulfur fuel oil sample taken at the loading port.

(iii) Together with the charterer, take two parallel samples (1.5 L × 2 canisters) from each ship tank carrying the low sulfur fuel oil for futures delivery in accordance with GB/T 4756, and properly seal and label the samples.

(c) Proportionally mixed samples from ship tanks (A2)

The Designated Inspection Agency shall prepare two sets (1.5 L × 2 canisters) of proportionally mixed samples based on Sample A1, and properly seal and label them.

(d) Shore tank sampling (Sample B)

Before unloading, take three parallel samples (3 L × 3 canisters) from each shore tank in accordance with GB/T 4756 and properly seal and label them. Where the innage is too low for the taking of Sample B, the Designated Inspection Agency shall note it down in the sampling report. The before-transfer density of the tank shall be the one in the latest inspection report provided by the Designated Delivery Storage Facility.

(e) The foregoing samples shall be sealed and stored at the location specified by the Designated Inspection Agency and, where needed for testing purposes, any set may be opened.

(4) Post-load-in shore tank inspection

(A) Measurement

(a) Take measurements four (4) hours after the oil surface has stabilized following unloading.

(b) Check the valve seals and residual oil in pipelines and make best effort to ensure the latter is in the same condition before and after oil transfer. If the conditions differ before and after unloading, the difference shall be accounted for in computations.

(c) Measure the innage or ullage, free water, oil temperature, and tank ambient temperature together with the measuring personnel of the Designated Delivery Storage Facility and the Consigner in accordance with the methods under Clause (2)(A)(e) of this Article 6.

(B) Sampling

(a) Shore tank sampling (Sample C)

After unloading, take three parallel samples (3 L × 3 canisters) from each shore tank in accordance with GB/T 4756 and properly seal and label them.

(b) The foregoing samples shall be sealed and stored at the location specified by the Designated Inspection Agency and, where needed for testing purposes, any set may be opened.

Article 7	As requested by GB/T 19779, weight of low sulfur fuel oil in shore tanks shall be calculated according to the density value from laboratory testing.

Article 8	Laboratory Testing at Load-In

(1) Designated Inspection Agencies shall conduct laboratory testing on the items prescribed by the Exchange (see Appendix I), which include kinematic viscosity, density, calculated carbon aromaticity index (CCAI), sulfur content, flash point (closed cup), hydrogen sulfide, acid value, total sediment (thermal aging test), carbon residue, pour point, moisture, ash content, vanadium, sodium, aluminum + silicon, net calorific value, used lubricating oil (ULO) (calcium and zinc, calcium and phosphorus), compatibility, cleanness, styrene, and phenol.

Styrene and phenol are tested through gas chromatography-mass spectrometry (GC-MS). See Appendix III for details.

(2) Any laboratory conducting these tests shall have the certifications recognized by the China National Accreditation Service for Conformity Assessment (CNAS).

(3) All samples shall be kept for three (3) months.

(4) Sample testing procedures.

(A) Sample C passes → testing ends.

(B) Sample C fails → Sample B fails → Sample A2 passes → testing ends.

(C) Sample C fails → Sample B passes → Sample A2 fails → testing ends.

(D) Samples C fails → Sample B fails → Sample A2 fails → testing ends.

(E) Sample C fails → Sample B passes → Sample A2 passes → testing on Sample A1 for each individual item that fails the test → testing ends.

The Designated Inspection Agency shall issue an inspection report upon the end of testing.

### Section 3	Load-Out Inspection

Article 9	Acceptance of Inspection Assignment by Designated Inspection Agencies

Consigner shall entrust, in writing and 24 hours before the load-out of low sulfur fuel oil for delivery, a Designated Inspection Agency to perform the load-out inspection. For the purpose of entrustment, the Consigner shall provide the weight value that is to be cancelled from the standard warrants for low sulfur fuel oil; the Designated Delivery Storage Facility shall provide the serial number of the relevant shore tanks and other relevant materials. The Designated Inspection Agency shall maintain close contact with the Consigner and the Designated Delivery Storage Facility, stay current with the load-out status, and timely arrange for inspections.

Article 10	On-Site Load-out Inspection by Designated Inspection Agencies

(1) Unsealing and resealing

(A) The Designated Inspection Agency shall ensure all valves to any shore tank that are not designated for futures delivery are tightly closed and properly sealed.

(B) Before the load-out, the Designated Inspection Agency shall, together with the Designated Delivery Storage Facility, jointly confirm the integrity of the seals on the designated futures shore tanks secured in place after the latest load-in or load-out, and verify the seal numbers. If there are no abnormalities, the Designated Inspection Agency shall unseal the outlet valves of the storage tanks and approve the shipment, and timely reseal them following the shipment. The Exchange shall be immediately notified of any issue uncovered during the verification process.

Where the owner does not entrust any Designated Inspection Agency, the person authorized by the Designated Delivery Storage Facility to handle futures delivery shall be responsible for arranging the unsealing and resealing. Such resealing shall be completed within 24 hours of the completion of load-out.

(2) Pre-load-out shore tank inspection

(A) Measurement

(a) Ascertain the qualifications of the shore tanks designated for low sulfur fuel oil delivery according set in JJG 168. Shore tanks shall be calibrated by a qualified national metrological administrative authority; the tank capacity tables shall be within the calibration validity period.

(b) Check the fullness of shore tank pipelines. The pipelines shall be fully filled to ensure accuracy in the measurement of the quantity of low sulfur fuel oil transferred. Designated Delivery Storage Facilities equipped with a circulation system shall activate it to fully fill the pipelines.

(c) Ascertain, in accordance with GB/T 13236, that all equipment used for shore tank measurement (including thermometers and gauging tapes) is calibrated by a qualified national metrological administrative authority, within the calibration validity period, and compliant with the explosion protection policies of the Designated Delivery Storage Facility.

(d) Measure the innage or ullage, free water, oil temperature, and tank ambient temperature together with the measuring personnel of the Designated Delivery Storage Facility and the Consigner in accordance with GB/T 13894 and GB/T 8927. If oil transfer is not started within eight (8) hours following the initial measurement, re-measurement according to the foregoing method shall be conducted and the results of which shall be conclusive.

(e) Measurements in shore tanks that have a low level of low sulfur fuel oil shall avoid the critical zone.

(B) Sampling

(a) Shore tank sampling

Before low sulfur fuel oil transfer, take three parallel samples (3 L × 3 canisters) from each shore tank in accordance with GB/T 4756 and properly seal and label them.

(b) The foregoing samples shall be sealed and stored at the location specified by the Designated Inspection Agency and, where needed for testing purposes, any set may be opened.

(3) Post-load-out shore tank inspection

(A) Take measurements two (2) hours after the oil surface has stabilized following the oil transfer.

(B) Check the valve seals and residual oil in pipelines and make best effort to ensure the latter is in the same condition before and after oil transfer. If the conditions differ, the difference shall be accounted for in computations.

(C) Measure the innage or ullage, free water, oil temperature, and tank ambient temperature together with the measuring personnel of the Designated Delivery Storage Facility and the Consigner in accordance with GB/T 13894 and GB/T 8927.

(D) Measurements in shore tanks that have a low level of low sulfur fuel oil shall avoid the critical zone.

Article 11	As requested in GB/T 19779, the weight of low sulfur fuel oil in shore tanks shall be calculated according to the density value from laboratory testing.

Article 12	Laboratory Testing at Load-Out

(1) Designated Inspection Agencies shall conduct laboratory testing on the items prescribed by the Exchange (see Appendix I), which include kinematic viscosity, density, CCAI, sulfur content, flash point (closed cup), hydrogen sulfide, acid value, total sediment (thermal aging test), carbon residue, pour point, moisture, ash content, vanadium, sodium, aluminum + silicon, net calorific value, used lubricating oil (ULO) (calcium and zinc, calcium and phosphorus), compatibility, cleanness, styrene, and phenol.

Styrene and phenol are tested through gas chromatography-mass spectrometry (GC-MS), see Appendix III for details.

(2) Any laboratory conducting these tests shall have the certifications recognized by CNAS.

(3) All samples shall be kept for three (3) months.

### Section 4	Pipeline Displacement

Article 13	Standards of Pipeline Displacement

(1) The pipeline displacement volume shall be based on the design capacity of the unloading pipeline between the vessel and the shore tanks as declared by the Designated Delivery Storage Facility, the volume of which shall be confirmed by the Designated Delivery Storage Facility and the Consigner with their signatures.

(2) To ensure measurement accuracy, the Designated Inspection Agency shall select no more than two (2) ship tanks that contain little or no free water for pipeline displacement, the selection of which shall be confirmed by the Designated Delivery Storage Facility and the Consigner with their signatures.

(3) The liquid level of the spot shore tanks for pipeline displacement shall avoid the critical zone.

(4) Any low sulfur fuel oil for futures delivery within the ship-to-shore tank pipelines shall be fully displaced into the designated futures shore tanks upon completion of unloading. The volume of low sulfur fuel oil loaded into such tanks shall be identical to the volume displaced through the pipelines since the start of unloading. Where discrepancy exists between the volume delivered from the ship and the volume received by the shore tanks, the latter shall be conclusive.

Article 14	Procedures for Displacing Residual Oil in Ship-to-Shore Tank Pipelines

(1) At the start of unloading, the residual oil within the ship-to-shore tank pipelines shall be displaced into spot shore tanks with the low sulfur fuel oil in designated ship tanks intended for futures delivery. The Designated Inspection Agency shall measure the liquid volume in the designated ship tanks and in the spot shore tanks both before and after pipeline displacement. The volume delivered from the designated ship tanks shall be applied in settlement.

(2) Upon completion of unloading, the low sulfur fuel oil for futures delivery within the ship-to-shore tank pipelines shall be displaced into designated futures shore tanks with the low sulfur fuel oil in spot shore tanks or in the designated ship tanks for the next shipment. The Designated Inspection Agency shall measure the liquid volume in the spot shore tanks or designated ship tanks and in the designated futures shore tanks both before and after the said displacement. The volume received by the designated futures shore tanks shall be applied in settlement. The details of this process are as follows:

(A) If the Designated Delivery Storage Facility is equipped with a circulation system, the displacement shall be conducted using the spot low sulfur fuel oil in spot shore tanks.

(B) If not, the displacement shall be conducted using the low sulfur fuel oil in the designated ship tanks for the next shipment.

Article 15	Any Designated Inspection Agency entrusted to oversee pipeline displacement shall measure the shore tanks and designated ship tanks involved in the displacement and issue a pipeline displacement report.

### Section 5	Inspection Report

Article 16	The results from any on-site load-in or load-out inspection shall be confirmed by the Consigner, the Designated Delivery Storage Facility, and the Designated Inspection Agency with signature. The Designated Inspection Agency shall issue the inspection report within two (2) days following the joint confirmation.

Rounding in calculations shall be governed by applicable standards and methods.

Article 17	An inspection report shall comprise a quality pre-check certificate, certificate of weight, shore tank measurement report, certificate of quality, sampling report, and pipeline displacement report (if necessary), among others. The unit of weight is either metric tons or kilograms.

Article 18	Designated Inspection Agencies shall issue the inspection report in both Chinese and English in accordance with the requirements of Consigners (see Appendix II for template). The report number shall be unique.

## Chapter 3	Miscellaneous

Article 19	For the purpose of these Rules, “low sulfur fuel oil” means low sulfur marine fuel oil that meets the corresponding quality standards set by the Exchange. “Low sulfur marine fuel oil” refers to homogeneous hydrocarbon mixtures extracted from petroleum, with certain amount of additives permitted for performance and characteristics gains. Low Sulfur marine fuel oils shall not contain any inorganic acid and used lubricating oil, any substance that may cause abnormal operations of ships, nor any artificially added additive or chemical waste that may endanger ship safety, adversely affect machine’s performance, or be harmful to health or increase air pollutions.

Any matter not covered by these Rules shall be governed by the Articles of Association, the General Exchange Rules, and other implementing rules of the Exchange.

Article 20	The Designated Inspection Agencies of the Exchange (jointly) reserve the right to interpret these Rules.

Article 21	These Rules shall come into effect as of Sep 18, 2020 after being filed with the Exchange.

Appendix I

# INE Low Sulfur Fuel Oil (Futures) Quality Standards

| Properties | Specifications | Test Method |
| --- | --- | --- |
| Kinematic viscosity (50 °C, mm2/s) | 380.0 max 100.0 min | ASTM D445 |
| Density (15 °C, kg/m3) | 991.0 max 930.0 min | ASTM D1298 |
| Calculated Carbon Aromaticity Index (CCAI) | 870 max | ISO 8217:2017(E) |
| Sulfur content (m/m, %) | 0.50 max | ASTM D4294 |
| Flash point (closed cup) (°C) | 60.0 min | ASTM D93 |
| Hydrogen sulfide (mg/kg) | 2.00 max | IP 570 |
| Acid value (mg KOH/g) | 2.5 max | ASTM D664 |
| Total sediment (thermal aging test) (m/m, %) | 0.10 max | ASTM D4870 |
| Carbon residue (m/m, %) | 18.00 max | ASTM D4530 |
| Pour point (°C) | 30 max | ASTM D97 |
| Moisture (V/V, %) | 0.50 max | ASTM D95 |
| Ash content (m/m, %) | 0.100 max | ASTM D482 |
| Vanadium (mg/kg) | 350 max | IP 501 |
| Sodium (mg/kg) | 100 max | IP 501 |
| Aluminum + Silicon (mg/kg) | 60 max | IP 501 |
| Net calorific value (cal/g) | 9,500 min | ASTM D240 |
| Used lubricating oil (ULO) (mg/kg) Calcium and Zinc Calcium and phosphorus | Fuel oil should be free of ULO, which is deemed to be present if any of the following conditions is met: Ca > 30 and Zn > 15 or Ca > 30 and P > 15 | IP 501 |
| Compatibility (level) | No higher than spot No. 2 | ASTM D4740 |
| Cleanness (level) | No higher than spot No. 2 | ASTM D4740 |
| Styrene (mg/kg) | 20 max | GB/T 6041 |
| Phenol (mg/kg) | 10 max | GB/T 6041 |

Appendix II

# Quality Pre-Check Report

(Bilingual)

No.:

Date:

Low Sulfur Fuel Oil (Futures) Inspection

## Quality Pre-Check Certificate

(Low Sulfur Fuel Oil Load-In)

Applicant:		XXXX

Declared product:		XXXX Low Sulfur fuel oil

Declared weight:		XXXX metric tons

Approval notice No.: 	XXXX

Designated Delivery Storage Facility:	XXXX

Sample source: 		Tank XX; ship xx, ship tank XX

Sample category:		Sample C, tank XX; or Sample A2, ship xx, ship tank XX

Sampling date:		YYYY/MM/DD - DD

Laboratory:

Testing results:

Our inspectors took representative samples in accordance with GB/T 4756, with the following testing results:

| Properties | Unit | Standard | Result | Test Method |
| --- | --- | --- | --- | --- |
| Density (15 °C) | kg/m3 | 991.0 max 930.0 min | XXX.X | ASTM D1298 |
| Kinematic viscosity (50 °C) | mm2/s | 380.0 max 100.0 min | XXX.X | ASTM D445 |
| Sulfur content | % (m/m) | 0.50 max | To 3 significant figures | ASTM D4294 |
| Moisture | % (v/v) | 0.50 max | X.X5 | ASTM D95 |
| Flash point (closed cup) | °C | 60.0 min | XX.5 | ASTM D93 |
| Compatibility | Level | No higher than spot No. 2 | X | ASTM D4740 |

Conclusion:

Our testing shows that the above low sulfur fuel oil product meets the quality requirements for the load-in pre-check as specified in the standard low sulfur fuel oil futures contract of Shanghai International Energy Exchange.

Note: This Certificate is issued in one original and three duplicates

* * * End * * *

# Inspection Report

No.:

Date:

Low Sulfur Fuel Oil (Futures) Inspection

## Certificate of Weight

(Low Sulfur Fuel Oil Load-In / Load-Out)

Applicant:		XXXX

Declared product:	XXXX Low Sulfur fuel oil

Declared weight:	XXXX metric tons

Approval notice No.: 	XXXX

Designated Delivery Storage Facility:	XXXX

Delivery tank No.:	Tank XX; Tank XX

Load-in / Load-out method:	Unload from XX to storage facility XX / From storage facility XX to XX

Inspection time:		YYYY/MM/DD - DD

Inspection standard:	Low Sulfur Fuel Oil (Futures) Inspection Rules (Trial)

Inspection results:

Based on measurements of ullage, free water, and oil temperature taken before and after load-out / load-in at the above shore tanks, the measurement tables provided by the shore tank operator, and appropriate corrections for density, our inspectors have calculated the load-in / load-out quantity for the abovementioned low sulfur fuel oil to be XXXX.XXX metric tons, or XXXX.0 kilograms. Details are as follows: Tank XX, load-in / load-out quantity of XXXX.XXX metric tons (XXXX.0 kg), (in)compliant with quality requirements; Tank XX, load-in / load-out quantity of XXXX.XXX metric tons (XXXX.0 kg), (in)compliant with quality requirements; Total compliant quantity: XXXX.XXX metric tons (XXXX.0 kg).

Note: This Certificate is issued in one original and three duplicates

Signature and seal of the Designated Inspection Agency:

* * * End * * *

No.:

Date:

Low Sulfur Fuel Oil (Futures) Inspection

## Shore Tank Measurement Report

| Designated Delivery Storage Facility |  | Declared product |  |
| --- | --- | --- | --- |
| Ship |  | Inspection date | YYYY/MM/DD - DD |

| Parameter | Tank No.: |  | Tank No.: |  |
| --- | --- | --- | --- | --- |
|  | Before | After | Before | After |
| Date and time | XXXX-XX-XX XX:XX |  |  |  |
| Liquid level (m) | .XXX |  |  |  |
| Free water level (m) | .XXX |  |  |  |
| Oil temperature (°C) | .X5 |  |  |  |
| Total measured height (m) | .XXX |  |  |  |
| Total observed volume (m3) | .XXX |  |  |  |
| Pipeline volume (m3) | .XXX |  |  |  |
| Free water volume (m3) | .XXX |  |  |  |
| CTE | .XXXXX |  |  |  |
| Floating roof adjustment (m3) | .XXX |  |  |  |
| Gross observed volume (m3) | .XXX |  |  |  |
| VCF (T 60B) | .XXXX |  |  |  |
| GSV (m3 @ 20 °C) | .XXX |  |  |  |
| Density (kg/m3 @ 20 °C) | .X |  |  |  |
| Gross fuel quantity (kg) | .0 |  |  |  |
| Gross fuel quantity (metric ton) | .XXX |  |  |  |

Low Sulfur Fuel Oil Transfer Quantity

| Tank No. |  |  |
| --- | --- | --- |
| Gross fuel quantity (kg) | .0 |  |
| Gross fuel quantity (m/t) | .XXX |  |

The above density value is based on testing by *** laboratory

* * * End * * *

No.:

Date:

Low Sulfur Fuel Oil (Futures) Inspection

## Certificate of Quality

(Low Sulfur Fuel Oil Load-In / Load-Out)

Applicant:		XXXX

Declared product:	XXXX Low Sulfur fuel oil		Declared weight: XXXX metric tons

Approval notice No.: 	XXXX

Designated Delivery Storage Facility:	XXXX

Sample source: 		Tank XX; Tank XX		Sampling date: YYYY/MM/DD - DD

Sample category:	Sample C or Sample B, tank XX

Laboratory:

Testing results:

Our inspectors took representative samples in accordance with GB/T 4756, with the following testing results:

| Properties | Unit | Specifications | Result | Test Method |
| --- | --- | --- | --- | --- |
| Kinematic viscosity (50 °C) | mm2/s | 380.0 max 100.0 min | XXX.X | ASTM D445 |
| Density (15 °C) | kg/m3 | 991.0 max 930.0 min | XXX.X | ASTM D1298 |
| CCAI |  | 870 max | 3 sig. figures | ISO 8217:2017(E) |
| Sulfur content | % (m/m) | 0.50 max | 3 sig. figures | ASTM D4294 |
| Flash point (closed cup) | °C | 60.0 min | XX.5 | ASTM D93 |
| Hydrogen sulfide | mg/kg | 2.00 max | X.XX | IP 570 |
| Acid value | mg KOH/g | 2.5 max | X.XX | ASTM D664 |
| Total sediment (thermal aging test) | % (m/m) | 0.10 max | X.XX | ASTM D4870 |
| Carbon residue | % (m/m) | 18.00 max | 3 sig. figures | ASTM D4530 |
| Pour point | °C | 30 max | Integer | ASTM D97 |
| Moisture | % (v/v) | 0.50 max | X.X5 | ASTM D95 |
| Ash content | % (m/m) | 0.100 max | X.XXX | ASTM D482 |
| Vanadium | mg/kg | 350 max | Integer | IP 501 |
| Sodium | mg/kg | 100 max | Integer | IP 501 |
| Aluminum + Silicon | mg/kg | 60 max | Integer | IP 501 |
| Net calorific value | cal/g | 9,500 min | Integer | ASTM D240 |
| ULO Calcium and Zinc Calcium and phosphorus | mg/kg | Fuel oil should be free of ULO, which is deemed to be present if any of the following conditions is met: Ca > 30 and Zn > 15 or Ca > 30 and P > 15 | Integer | IP 501 |
| Compatibility | Level | No higher than spot No. 2 | X | ASTM D4740 |
| Cleanness | Level | No higher than spot No. 2 | X | ASTM D4740 |
| Styrene | mg/kg | 20 max | Integer | GB/T 6041 |
| Phenol | mg/kg | 10 max | Integer | GB/T 6041 |

Conclusion:

Our testing shows that the above Low Sulfur fuel oil product meets the quality requirements as specified in the standard Low Sulfur fuel oil futures contract of Shanghai International Energy Exchange.

Note: This Certificate is issued in one original and three duplicates

* * * End * * *

No.:

Date:

Low Sulfur Fuel Oil (Futures) Inspection

## Sampling Report

| Designated Delivery Storage Facility |  | Declared product |  |
| --- | --- | --- | --- |
| Ship |  | Inspection date |  |
| Sampling agency |  | Sampling inspector |  |

| Category | Source | Quantity | Use | Type | Sampling Time | Seal Number |
| --- | --- | --- | --- | --- | --- | --- |
| C | Shore tank XX | 1 × 3 L | Lab analysis | Mixture |  | X |
| C | Shore tank XX | 2 × 3 L | Seal and store | Mixture |  | X/X |
| B | Shore tank XX | 1 × 3 L | Lab analysis | Mixture |  | X |
| B | Shore tank XX | 2 × 3 L | Seal and store | Mixture |  | X/X |
| A1 | Ship tank #1 | 2 × 1.5 L | Seal and store | Mixture |  | X/X |
| A1 | Ship tank #2 | 2 × 1.5 L | Seal and store | Mixture |  | X/X |
| A1 | … | 2 × 1.5 L | Seal and store | Mixture |  | X/X |

Note: Samples should be kept for three months. In the event of a dispute extending beyond the three-month period, the samples should be kept until the dispute is resolved.

* * * End * * *

No.:

Date:

Low Sulfur Fuel Oil (Futures) Inspection

## Pipeline Displacement Report

| Designated Delivery Storage Facility |  | Declared product |  |
| --- | --- | --- | --- |
| Ship |  | Inspection date | YYYY/MM/DD - DD |

| Parameter | Delivering Ship Tank No. |  |  |  | Receiving Shore Tank No. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | XX |  | XX |  | XX |  | XX |  |
|  | Before | After | Before | After | Before | After | Before | After |
| Date and time |  |  |  |  |  |  |  |  |
| Liquid level (m) | .XXX |  |  |  |  |  |  |  |
| Free water level (m) | .XXX |  |  |  |  |  |  |  |
| Oil temperature (°C) | .X5 |  |  |  |  |  |  |  |
| Total measured height (m) | .XXX |  |  |  |  |  |  |  |
| Total observed volume (m3) | .XXX |  |  |  |  |  |  |  |
| Pipeline volume (m3) | .XXX |  |  |  |  |  |  |  |
| Free water volume (m3) | .XXX |  |  |  |  |  |  |  |
| Floating roof adjustment (m3) | .XXX |  |  |  |  |  |  |  |
| Gross observed volume (m3) | .XXX |  |  |  |  |  |  |  |
| VCF (T 60B) | .XXXX |  |  |  |  |  |  |  |
| GSV (m3 @ 20 °C) | .XXX |  |  |  |  |  |  |  |
| Density (kg/m3 @ 20 °C) | .X |  |  |  |  |  |  |  |

Delivered / Received Low Sulfur Fuel Oil Volume

| Ship tank No. / Shore tank No. | XX | XX |
| --- | --- | --- |
| TOV (m3 @ observed temp.) | .XXX | .XXX |
| GSV (m3 @ 20 °C) | .XXX | .XXX |

Notes:

1. “Liquid level” for “Delivering Ship Tank No.” column refers to the ullage of the tank.

2. The design capacity of pipelines as declared by the Designated Delivery Storage Facility is 		 m3.

* * * End * * *

Appendix III

Gas Chromatography and Mass Spectrometry Testing of Styrene and Phenol in Low Sulfur Fuel Oil

1.	Scope

The gas chromatography and mass spectrometry (GC-MS) method is applicable to the testing of the amount of styrene and phenol in Low Sulfur fuel oil.

Low Sulfur fuel oil to be tested shall be homogenous and without stratification and sediment.

2.	Normative Reference Document

GB/T 6041 – General Rules for Mass Spectrometric Analysis

3.	Terms and Definition

| No. | CAS Registry Number | English Name | Chinese Name | Limit (mg/kg) |
| --- | --- | --- | --- | --- |
| 1 | 100-42-5 | Styrene | benyixi | ≤ 20 |
| 2 | 108-95-2 | Phenol | benfen | ≤ 10 |

4.	Summary

Dissolve the sample in a toluene-ethanol mixture (4:1 by volume), filter it through an organic membrane, and perform a GC-MS analysis. Conduct a qualitative identification of styrene and phenol based on a total ion chromatogram (TIC) and gas spectrum (GS), and quantitative calculation through selected ion monitoring (SIM).

5.	Application

Styrene and phenol have significant adverse effects on the quality of marine low sulfur fuel oil, including coking, erosion, stratification, and thus affect its normal use. GC-MS analysis is an accurate and efficient method to determine the amount of styrene and phenol in marine low sulfur fuel oil.

6.	Interference

The characteristic ions of styrene are detected at m/z 104, which is shared by some hydrocarbons in low sulfur fuel oil. Therefore, quantitative calculation should be made in view of the chromatographic retention time and reference mass spectrum of standard styrene, so as to ensure that ions at m/z 104 are produced by styrene, not hydrocarbons.

7.	Reagents and materials

Unless otherwise specified, all reagents used in this method should be chromatographically pure.

7.1	Toluene

7.2	Ethanol

7.3	Styrene

7.4	Phenol

7.5	Carrier gas: helium, 99.999% pure or higher.

7.6	Volumetric flasks: 20 mL, 50 mL.

7.7	Standard stock solution: Weigh and mix an appropriate amount of standard styrene and phenol in a 50 mL volumetric flask, add the toluene-ethanol mixture (4:1 by volume) into the flask to prepare standard stock solution with a concentration of 50 mg/mL.

Note: The standard stock solution is preferably kept in a refrigerator (0-4 °C) for use within six months.

7.8	Standard testing solution: prepare standard testing solution with a concentration of 10 mg/L to 100 mg/L as needed through stepwise dilution with the 4:1 toluene-ethanol mixture.

Note: standard testing solution is preferably kept in a refrigerator (0-4 °C) for use within three months.

7.9	Organic filter membrane: PTFE, with a pore size of 0.45 μm.

8.	Instruments and Equipment

8.1	GC-MS instrument with EI ion source.

8.2	Balance with a resolution of 0.1 mg.

9.	Analytical Procedures

9.1	Sample preparation

Weigh a 2 g sample of low sulfur fuel oil to the nearest 0.1 mg, fully mix and dissolve the sample in the 4:1 toluene-ethanol mixture to a volume of 20 mL, filter the solution through a 0.45 μm organic membrane to produce a sample solution, then perform a GC-MS analysis.

9.2	Blank test

Process reagents to be used in the test according to 9.1, then perform a GC-MS analysis.

9.3	Determination

9.3.1	Specifications for GC-MS instrument

Because GC-MS instruments vary in their specifications, no universal values can be given. Nonetheless, the specifications of an instrument should ensure that during the GC-MS testing, the components under testing can be readily distinguished from the other components (mainly hydrocarbons). The specifications listed below are for reference.

| Volume injection | 1.0 μL |
| --- | --- |
| Chromatography column | Restek DHA-50, 50 m × 200 μm × 0.5 μm or high-temperature-resistant, non-polar chromatography columns with comparable specifications |
| Column heating process | Hold at an initial temperature of 50 °C for 1 minute, heat the column to 100 °C at 20 °C/min and hold for 5 minutes, then heat to 300 °C at 20 °C/min and hold for 10 minutes |
| Column flow (constant flow) | 2 mL/min |
| Injection port | Split-flow mode, 10:1 split ratio, 300 °C |
| Transmission line temperature | 300 °C |
| Type of ion source | Electron ionization (EI) |
| MS tune method | Automatic tune |
| MS scan mode | Scan, selected ion monitoring (SIM) |
| MS scan range | 50-700 |
| Characteristic ion | Styrene: m/z 104 phenol: m/z 94 |
| Time of operation | 28.5 min |

9.3.2	Qualitative analysis

First, determine the retention time for styrene and phenol in the standard stock solution with GC-MS, then test the sample solution with the GC-MS instrument under the same settings and conditions. If a chromatographic peak is detected within the retention time and the corresponding fragmentation spectrum is identical to the standard fragmentation spectra of styrene and phenol, then the sample is deemed to contain styrene and phenol.

9.3.3	Quantitative analysis

Test the standard testing solutions (as described in 7.8) of at least five different concentrations, then make a standard plot of peak area versus concentration. A sample solution with a concentration beyond the linear range should be diluted before testing.

This method uses external standard quantification for styrene and phenol through integration of the peak area of characteristics ions according to 9.3.1.

10.	Formula

Read the peak area of the characteristic ions of styrene and phenol in the sample solution, identify the concentrations of styrene and phenol Ci based on the standard plot, and calculate the amount of styrene and phenol according to equation (1).

(1)

where:

X0 is the amount (mg/kg) of styrene or phenol in the sample;

Ci is the concentration (mg/L) of styrene or phenol in the sample solution;

V is the standard volume (mL) of the sample;

m is the mass (g) of the sample.

The result should be rounded to three significant figures.

11.	Quantitative Limit, Recovery Rate, and Precision

11.1	Quantitative limit

The quantitative limits for styrene and phenol are both 10 mg/kg.

11.2	Recovery rate

Add a certain amount of standard solution of known concentration to the sample, and analyze the recovery rate according to the steps under section 9. The recovery rate should be between 90% and 110%.

11.3	Precision

The absolute difference between two independent measurements conducted under repeatability conditions should not exceed 10% of the average value.

12.	Testing report

The testing report should at least include:

a. Source and description of the sample:

b. Testing result: average value of the parallel samples;

c. Abnormalities observed during the testing; and

d. Testing date.